root@demo-armotec-quectel-lte-5g:~# cat /usr/lib/python3/dist-packages/tveif/if_lte.py

#!/usr/bin/env python3

# -*- coding: utf-8 -*-

from __future__ import print_function

import os

import subprocess

import sys

import xml.etree.ElementTree

from collections import defaultdict

from configobj import ConfigObj

from systemd import journal

import re

import time

import serial

if 'eventlet.os' in sys.modules:

lteStream = sys.stdout

else:

lteStream = journal.stream(identifier='if_lte')

apn_dbase = '/scripts/apn/apns-conf.xml'

apn_mcc_file = '/scripts/apn/apncountrycode.txt'

apn_default = "internet"

user_default = ""

pass_default = ""

modem_default = '/dev/cdc-wdm0'

mbim_modem_default= '/dev/cdc-wdm_pcie'

modem_tty_default = '/dev/ttyUSB2'

QMI_DRV = 1

MHI_DRV = 2

CDC_DRV = 3

MBM_DRV = 4

MBIM_CANDIDATES = [

"/dev/cdc-wdm_pcie",

"/dev/cdc-wdm0",

]

def get_mcc_codes(dev):

mcc = "0"

mnc = "0"

try:

output = subprocess.check_output(['/usr/bin/qmicli', '-d', dev, '--nas-get-home-network'])

except:

return int(mcc), int(mnc)

for ln in output.splitlines():

if bytes('MCC', encoding='utf8') in ln:

fields = ln.split(b"'")

mcc = fields[1]

if bytes('MNC', encoding='utf8') in ln:

fields = ln.split(b"'")

mnc = fields[1]

if bytes('Description', encoding='utf8') in ln:

fields = ln.split(b"'")

return int(mcc), int(mnc)

def get_lte_apn(apn_file, mcc_file, mcc, mnc):

codes = defaultdict(dict)

with open(mcc_file, encoding="ISO-8859-1") as f:

for x in f:

code, country = x.rstrip().split(None, 1)

codes.update({int(code): country})

apns = xml.etree.ElementTree.parse(apn_file).getroot()

if 0 != mcc:

for acr in apns.findall('apn'):

if int(acr.get('mcc')) == mcc and int(acr.get('mnc')) == mnc:

if "default" in acr.get('type'):

apn = acr.get('apn')

user = acr.get('user')

password = acr.get('password')

if not apn:

apn = apn_default

if not user:

user = user_default

if not password:

password = pass_default

return apn, user, password

print("Local operator %d,%d in '%s' not in our database,going to use default apn '%s'" %

(mcc, mnc, codes.get(mcc), apn_default), file=lteStream)

else:

print("Failed to get mcc code,going to use default apn '%s'" % apn_default, file=lteStream)

return apn_default, user_default, pass_default

def get_qmi_authenticators(if_cfg, modem):

if 'apn' not in if_cfg or not if_cfg['apn']:

mcc, mnc = get_mcc_codes(modem)

apn, user, password = get_lte_apn(apn_dbase, apn_mcc_file, mcc, mnc)

else:

apn = if_cfg['apn'] if 'apn' in if_cfg else ""

user = if_cfg['user'] if 'user' in if_cfg else ""

password = if_cfg['pass'] if 'pass' in if_cfg else ""

return apn, user, password

def lte_is_qmi(modem_drv):

qmi_missing = os.system('ls %s 2>/dev/null >/dev/null' % modem_drv)

if qmi_missing!=0:

return False

if lte_get_driver_type_by_dev(modem_drv) != QMI_DRV:

return

qmi_noop=os.system("/usr/bin/qmicli -d %s --device-open-qmi --dms-noop" % modem_drv)

if qmi_noop==0:

return True

return False

def lte_is_mbim(modem_drv=None):

modem_drv = mbim_dev_check(modem_drv)

if not modem_drv:

return False

drv_missing = os.system('ls %s 2>/dev/null >/dev/null' % modem_drv)

if drv_missing!=0:

return False

mbim_noop=os.system("/usr/bin/mbimcli -d %s -p  --no-close --noop" % modem_drv)

if 0==mbim_noop:

return True

return False

def lte_qmi_sim_ready(modem_drv):

retcode = False

out = ""

try:

out = subprocess.check_output(['/usr/bin/qmicli', '-d', modem_drv, '--uim-get-card-status']).decode()

except:

pass

if 'ready' in out.lower():

retcode = True

return retcode

def lte_qmi_is_connected(modem_drv):

retcode = False

out = ""

try:

out = subprocess.check_output(['/usr/bin/qmicli', '-d', modem_drv, '--wds-get-packet-service-status']).decode()

except:

pass

m = re.match(".*Connection status:.*'(.*)'.*", out)

if not m:

#we got timeout or other message , try with sync option

try:

out = subprocess.check_output(['/usr/bin/qmicli', '-d', modem_drv, '--wds-get-packet-service-status', '--device-open-sync']).decode()

except:

pass

m = re.match(".*Connection status:.*'(.*)'.*", out)

if m and m.group(1):

connst = m.group(1)

if connst.lower() == "connected":

retcode = True

return retcode

def lte_mbim_is_connected(modem_drv=None):

modem_drv = mbim_dev_check(modem_drv)

if not modem_drv:

return False

retcode = False

out = ""

try:

out = subprocess.run(['/usr/bin/mbim-network', modem_drv, 'status'], check=False, stdout=subprocess.PIPE, stderr=subprocess.PIPE).stdout.decode()

except:

return False

m = re.search(".*Status: activated.*", out)

if m:

retcode = True

return retcode

def save_lte_qmi_pdh(modem, apn, user, password, qmi_output):

pdh = ""

cid = ""

pdh_match = re.search('.*handle:\s*(.*)\s*', str(qmi_output))

cid_match = re.search('.*CID:\s*\'(.*)\'\s*', str(qmi_output))

if pdh_match is not None and pdh_match.group(1) is not None:

pdh = pdh_match.group(1)

pdh = pdh.strip('\'"')

if cid_match is not None and cid_match.group(1) is not None:

cid = cid_match.group(1)

lte_inf = ConfigObj()

lte_inf['pdh'] = pdh

lte_inf['cid'] = cid

lte_inf['apn'] = apn

lte_inf['user'] = user

lte_inf['password'] = password

lte_inf.filename = "/run/" + os.path.basename(modem) + ".prof"

lte_inf.write()

def lte_putcmd(modem, cmd):

print('Modem command: %s' % cmd, file=lteStream)

modem.write((cmd + '\r').encode())

time.sleep(0.5)

def lte_getresponse(modem, maxlines=3, timeout=1):

buf = ''

while True:

modem.timeout = timeout

l = modem.readline()

if not l:

break

buf += l

maxlines -= 1

if maxlines <= 1:

break

print('Modem response %s' % buf, file=lteStream)

return buf

def reset_modem_profile(tty_dev):

try:

modem_tty = serial.Serial(tty_dev, 115200, timeout=3)

lte_putcmd(modem_tty, "AT+CGDCONT=1,\"IP\"")

time.sleep(1)

lte_getresponse(modem_tty)

modem_tty.close()

time.sleep(3)

except:

pass

def set_modem_online(tty_dev):

try:

modem_tty = serial.Serial(tty_dev, 115200, timeout=3)

lte_putcmd(modem_tty, "AT+CFUN=1")

time.sleep(1)

lte_getresponse(modem_tty)

modem_tty.close()

time.sleep(3)

except:

pass

def reset_lte_qmi_dev(modem):

if not os.path.exists(modem):

print('No device %s found' % modem, file=lteStream)

return

try:

subprocess.check_output(['/usr/bin/qmicli', '-d', modem, '--dms-set-operating-mode=offline'])

time.sleep(5)

subprocess.check_output(['/usr/bin/qmicli', '-d', modem, '--dms-set-operating-mode=reset'])

time.sleep(30)

except:

pass

def setup_lte_qmi_dev(if_cfg, modem):

if not os.path.exists(modem):

print('No device %s found' % modem, file=lteStream)

return

#if if_cfg['name'] != "lte0":

#    return

if lte_get_driver_type(if_cfg['name']) != QMI_DRV:

return

os.system("/usr/bin/qmicli -d %s --uim-sim-power-on=1" %modem)

ip_type = "4"

pdp_type="ipv4"

if 'bootproto' in if_cfg and ('dhcp6' in if_cfg['bootproto'] or 'dhcp46' == if_cfg['bootproto']):

ip_type="6"

pdp_type="ipv6"

#store list of networks

if not os.path.exists("/run/ltescan.txt"):

os.system("/usr/bin/timeout 60s /usr/bin/qmicli -d %s --nas-network-scan > /run/ltescan.txt" %modem)

auth = ""

apn, user, password = get_qmi_authenticators(if_cfg, modem)

if os.path.exists(modem_tty_default):

reset_modem_profile(modem_tty_default)

time.sleep(2)

start_cmd = ['/usr/bin/qmicli', '-d', modem,

'--wds-start-network=%s,%s,%s,%s,%s' % (apn, auth, user, password, ip_type),

'--client-no-release-cid', '--verbose', '--device-open-sync']

if user!="" and password!="":

custom_cmd='/usr/bin/qmicli -d %s --wds-start-network="apn=%s,ip-type=%s,username=%s,password=%s" --client-no-release-cid --verbose --device-open-sync' % (modem,apn, ip_type, user, password)

retcode = os.system(custom_cmd)

if retcode == 0:

print("Connected with cmd %s." % custom_cmd, file=lteStream)

return True

if user!="" or password!="" or apn!="":

mcc, mnc = get_mcc_codes(modem)

is_verizon = False

if mcc == 311 and mnc == 480:

#detect if we have Verizon with 2 profiles

profiles_1_and_3 = os.system('qmicli -d %s --wds-get-profile-list=3gpp | egrep "\[1\]|\[3\]" -c | grep -q "2"' % modem)

if profiles_1_and_3 == 0:

is_verizon = True

if apn.lower()=="vzwinternet" and is_verizon:

#set pfofiles 1 and 3

os.system('qmicli -d %s --wds-modify-profile="3gpp,1,apn=\'%s\'"' % (modem,"ims") )

os.system('qmicli -d %s --wds-modify-profile="3gpp,3,apn=\'%s\'"' % (modem,apn.upper()) )

os.system('qmicli -d %s --wds-modify-profile="3gpp,3,pdp-type=\'%s\'"' % (modem,pdp_type) )

if user!="":

os.system('qmicli -d %s --wds-modify-profile="3gpp,1,username=\'%s\'"' % (modem,user) )

os.system('qmicli -d %s --wds-modify-profile="3gpp,3,username=\'%s\'"' % (modem,user) )

if password!="":

os.system('qmicli -d %s --wds-modify-profile="3gpp,1,password=\'%s\'"' % (modem,password) )

os.system('qmicli -d %s --wds-modify-profile="3gpp,3,password=\'%s\'"' % (modem,password) )

os.system('qmicli -d %s --wds-set-default-profile-number=3gpp,3' % modem )

else:

#try to set profile 1

os.system('qmicli -d %s --wds-delete-profile=3gpp,1' % modem )

if os.system('qmicli -d %s --wds-swi-create-profile-indexed="3gpp,1,apn=\'%s\',pdp-type=\'%s\',username=\'%s\',password=\'%s\'"' % (modem,apn, pdp_type, user, password) ) !=0:

os.system('qmicli -d %s --wds-modify-profile="3gpp,1,pdp-type=\'%s\'"' % (modem,pdp_type) )

if apn!="":

os.system('qmicli -d %s --wds-modify-profile="3gpp,1,apn=\'%s\'"' % (modem,apn) )

if user!="":

os.system('qmicli -d %s --wds-modify-profile="3gpp,1,username=\'%s\'"' % (modem,user) )

if password!="":

os.system('qmicli -d %s --wds-modify-profile="3gpp,1,password=\'%s\'"' % (modem,password) )

os.system('qmicli -d %s --wds-set-default-profile-number=3gpp,1' % modem )

else:

os.system('qmicli -d %s --wds-delete-profile=3gpp,1' % modem )

time.sleep(3)

try:

print("attempt to init LTE with apn '%s'" % apn, file=lteStream)

out = subprocess.check_output(start_cmd, stderr=subprocess.STDOUT).decode()

print(out, file=lteStream)

save_lte_qmi_pdh(modem, apn, user, password, out)

return True

except subprocess.CalledProcessError as e:

print(e.output, file=lteStream)

start_cmd = ['/usr/bin/qmicli', '-d', modem,

'--wds-start-network="%s,%s,%s,%s,%s"' % (apn, auth, user, password, ip_type),

'--client-no-release-cid', '--verbose', '--device-open-sync']

if "translated = [ type = '3gpp' reason = '50' ]" in str(e.output) or "ipv4-only-allowed" in str(e.output):

start_cmd[3] = '--wds-start-network=\"'

if apn != "":

start_cmd[3] += "apn='%s'," % apn

if user != "":

start_cmd[3] += "username='%s'," % user

if password != "":

start_cmd[3] += "password='%s'," % password

start_cmd[3] += 'ip-type=4\"'

print(

"Failed - operator doesn't support ipv6. attempt to init LTE %s with apn '%s', authentication '%s', user '%s', password '%s', and ip type '%s'" %

(modem, auth, apn, user, password, ip_type), file=lteStream)

retcode = os.system('/usr/bin/qmicli -d %s %s --client-no-release-cid --verbose --device-open-sync 1>/dev/null 2>/dev/null' %

(modem, start_cmd[3]) )

if retcode == 0:

print("Connected.", file=lteStream)

return True

print("Failed to connect %s using apn '%s', authentication '%s', user '%s', password '%s', and ip type '%s'" % (

modem, apn, auth, user, password, ip_type), file=lteStream)

out = ""

time.sleep(3)

try:

out = subprocess.check_output(

['/usr/bin/qmicli', '-d', modem, '--wds-start-network=', '--client-no-release-cid', '--verbose',

'--device-open-sync'], stderr=subprocess.STDOUT).decode()

save_lte_qmi_pdh(modem, apn, user, password, out)

print("Connected.", file=lteStream)

return True

except subprocess.CalledProcessError:

print('Failed to connect without APN, user and password', file=lteStream)

return False

return False

except OSError as e:

print('Error -- %s' % repr(e), file=lteStream)

return False

#def mbim_dev_check(modem=mbim_modem_default):

#if os.path.exists(modem):

#   return modem

#if os.path.exists(modem_default):

#   return modem_default

def mbim_dev_check(candidates=MBIM_CANDIDATES):

# ако подадат string → правим го list

if isinstance(candidates, str):

candidates = [candidates]

for dev in candidates:

if dev and os.path.exists(dev):

return dev

print("No MBIM device found (checked: %s)" %

", ".join(candidates), file=lteStream)

return None

def setup_lte_mbim_dev(if_cfg, modem=None):

modem = mbim_dev_check(modem)

if not modem:

return

retry = False

if not os.path.exists("/run/ltescan.txt"):

os.system("/usr/bin/timeout 180s /usr/bin/mbimcli -d /dev/cdc-wdm0 -p --query-visible-providers > /run/ltescan.txt 2>/dev/null")

#delete empty scan result

os.system("find /run/ -size 0 -name ltescan.txt -delete")

if not os.path.exists(modem):

print('No device %s found' % modem, file=lteStream)

return

drv_type = lte_get_driver_type(if_cfg['name'])

if drv_type != MHI_DRV and drv_type != MBM_DRV:

return

if os.path.exists(modem_tty_default):

set_modem_online(modem_tty_default)

apn = if_cfg['apn'] if 'apn' in if_cfg else ""

user = if_cfg['user'] if 'user' in if_cfg else ""

password = if_cfg['pass'] if 'pass' in if_cfg else ""

start_cmd = ['/usr/bin/mbim-network',  modem, 'start']

with open('/etc/mbim-network.conf', 'w') as f:

if apn != "":

f.write("APN=%s\n" % apn)

else:

f.write("APN=internet\n" )

if user !="" and password !="":

f.write("APN_USER=%s APN_PASS=%s\n" %(user,password))

f.write("APN_AUTH=pap\n")

f.write("PROXY=yes\n")

try:

print("attempt to init LTE ", file=lteStream)

out = subprocess.check_output(start_cmd, stderr=subprocess.STDOUT).decode()

print(out, file=lteStream)

except subprocess.CalledProcessError as e:

print(e.output, file=lteStream)

retry = True

if retry:

time.sleep(3)

try:

out = subprocess.check_output(start_cmd, stderr=subprocess.STDOUT).decode()

except subprocess.CalledProcessError as e:

print(e.output, file=lteStream)

netup_cmd = ['/scripts/mbim-set-ip.sh',  modem, 'br-'+if_cfg['name'], str(if_cfg['metric']) if 'metric' in if_cfg else "110"]

try:

print("attempt to init LTE network stack", file=lteStream)

out = subprocess.check_output(netup_cmd, stderr=subprocess.STDOUT).decode()

print(out, file=lteStream)

except subprocess.CalledProcessError as e:

print(e.output, file=lteStream)

return False

def disconnect_lte_qmi_dev(modem, pdh):

if pdh is None or pdh == "":

return

stop_cmd = ['/usr/bin/qmicli', '-d', modem, '--wds-stop-network=%s' % pdh]

for _ in range(3):

try:

subprocess.check_output(stop_cmd, stderr=subprocess.STDOUT).decode()

except subprocess.CalledProcessError:

time.sleep(2)

if not lte_qmi_is_connected(modem):

break

try:

subprocess.check_output(stop_cmd, stderr=subprocess.STDOUT).decode()

except:

pass

time.sleep(5)

def load_lte_qmi_pdh(modem):

apn = ""

user = ""

password = ""

pdh = ""

cid = ""

lte_inf = ConfigObj("/run/" + os.path.basename(modem) + ".prof")

if 'pdh' in lte_inf:

pdh = lte_inf['pdh']

if 'cid' in lte_inf:

cid = lte_inf['cid']

if 'apn' in lte_inf:

apn = lte_inf['apn']

if 'user' in lte_inf:

user = lte_inf['user']

if 'password' in lte_inf:

password = lte_inf['password']

return apn, user, password, pdh, cid

def reconnect_lte_qmi_dev(if_cfg, modem, force=False):

if not lte_is_qmi(modem):

return

if not os.path.exists(modem):

return

apn, user, password = get_qmi_authenticators(if_cfg, modem)

(oldapn, olduser, oldpassword, pdh, cid) = load_lte_qmi_pdh(modem)

if force is False:

if not lte_qmi_is_connected(modem):

return

if apn == oldapn and user == olduser and password == oldpassword:

return

if pdh is None or pdh == "":

return

disconnect_lte_qmi_dev(modem, pdh)

setup_lte_qmi_dev(if_cfg, modem)

def lte_check_raw_if(if_name):

if not os.path.exists('/sys/class/net/' + if_name + '/qmi/raw_ip'):

if os.path.exists('/sys/class/net/' + if_name + '/type'):

raw_type = open('/sys/class/net/' + if_name + '/type', 'rb').readline().rstrip().decode()

if raw_type == '519':

return True

return False

raw_mode = open('/sys/class/net/' + if_name + '/qmi/raw_ip', 'rb').readline().rstrip().decode()

if raw_mode == 'Y':

return True

return False

def lte_get_raw_mac(ifname):

fake_mac = '00:00:00:00:a1:00'

def extract_mac_from_imei(imei_line):

try:

imei = imei_line.split(':')

imei_int = int(imei[-1].replace(' ', '').replace('\'', ''))

(_, mac) = divmod(imei_int, 0x1000000000000)

mac_str = '{:012x}'.format(mac)

octet1 = int(mac_str[0:2], 16) & 0xfe

octet1_str = "%02X" % octet1

return '{}:{}:{}:{}:{}:{}'.format(octet1_str, mac_str[2:4], mac_str[4:6], mac_str[6:8], mac_str[8:10],

mac_str[10:12])

except:

return fake_mac

if not os.path.exists('/sys/class/net/' + ifname + '/qmi'):

if os.path.exists('/dev/cdc-wdm_pcie'):

try:

out = subprocess.check_output(['mbimcli', '--device=/dev/cdc-wdm_pcie', '--device-open-proxy' , '--query-device-caps']).decode()

for line in out.split('\n'):

if 'Device ID' in line:

return extract_mac_from_imei(line)

except:

pass

return fake_mac

return fake_mac

try:

out = subprocess.check_output(['qmicli', '-d', '/dev/cdc-wdm' + ifname.strip('lte'), '--dms-get-ids']).decode()

for line in out.split('\n'):

if 'IMEI' in line:

return extract_mac_from_imei(line)

except:

return fake_mac

#Get type of the modem driver per interface name

#Input: name of the interface , i.e.: lte0

#Output: 1 - QMI ; 2 - MBIM ; 3 - CDC-Ether (dongle) ; 10 - Unknown

def lte_get_driver_type(ifname):

retcode = 10

out = ""

try:

out = subprocess.run(['/sbin/ethtool', '-i', ifname], check=False, stdout=subprocess.PIPE, stderr=subprocess.PIPE).stdout.decode()

except:

return retcode

if re.search(".*driver: mhi.*", out):

retcode=MHI_DRV

elif re.search(".*driver: qmi_wwan.*", out):

retcode=QMI_DRV

elif re.search(".*driver: cdc_ether.*", out):

retcode=CDC_DRV

elif re.search(".*driver: cdc_mbim.*", out):

retcode=MBM_DRV

return retcode

def lte_get_driver_type_by_dev(devpath):

retcode = 10

out = ""

devname=devpath.replace("/dev/","")

uevent_name='/sys/class/usbmisc/' + devname + '/device/uevent'

if not os.path.exists(uevent_name):

uevent_name='/sys/class/mhi_uci/' + devname + '/device/uevent'

if not os.path.exists(uevent_name):

return retcode

try:

out = subprocess.run(["cat", uevent_name], check=False, stdout=subprocess.PIPE, stderr=subprocess.PIPE).stdout.decode()

except:

return retcode

if re.search(".*DRIVER=mhi.*", out):

retcode=MHI_DRV

elif re.search(".*DRIVER=qmi_wwan.*", out):

retcode=QMI_DRV

elif re.search(".*DRIVER=cdc_ether.*", out):

retcode=CDC_DRV

elif re.search(".*DRIVER=cdc_mbim.*", out):

retcode=MBM_DRV

return retcode

root@demo-armotec-quectel-lte-5g:~#